Global Precipitation Mission (GPM)

Ground Validation System

GPM Validation Network Volume Matching Script Descriptions and Execution

August, 2017

Goddard Space Flight Center

Greenbelt, Maryland 20771

Document History

| Document Version | Date | Changes |
| --- | --- | --- |
| Draft | November, 2016 | Initial draft |
| 2nd draft | August, 2017 | Modified descriptions of script option PARAMETER_SET in relation to versions of IDL batch files used to configure matchups. Added discussion of related script do_DPR_GeoMatch_NPOL_OLY_rules.sh to the descriptions in Section 3.1. |

# Table of Contents

1.	Introduction	4

2.	Script Summaries – Baseline Matchups	5

2.1	do_DPR_GeoMatch.sh	5

2.2	do_DPRGMI_GeoMatch.sh	9

2.2.1	do_DPRGMI_GeoMatch_from_ControlFiles.sh	12

2.3	do_GMI_GeoMatch.sh	15

2.4	do_GR_HS_MS_NS_GeoMatch.sh	18

2.5	do_DPR2GR_GeoMatch.sh	21

3.	Script Summaries - Non-routine and legacy matchup scripts	24

3.1	do_DPR_GeoMatch_NPOL_MD_rules.sh	24

3.2	do_DPRtoGMI_OrbitMatch.sh	28

3.3	do_PR_GeoMatch.sh	30

3.4	doTMIGeoMatch4NewRainCases.sh	33

# Introduction

The Global Precipitation Measurement (GPM) Validation Network (VN) performs volume matches of data from the satellite-borne TRMM Precipitation Radar (PR), GPM Dual-frequency Precipitation Radar (DPR) and GPM, TRMM and constellation satellite Microwave Imagers (GMI, TMI, etc.) to coincident ground-based scanning weather radar (GR) observations for rain events occurring at the ground radar sites.

As part of the data ingest process of PR and DPR data, the VN software pre-analyzes the satellite radar products over the ground radars within the satellite data swath to determine the areal extent of precipitation of each rain type within 100 km range of the GR site, and stores this information as raw metadata in the VN PostGRESQL database ‘gpmgv’, in the ‘event_meta_numeric’ table.  This part happens automatically with no manual intervention required. A sequence of SQL queries in the file rainCases100kmAddNewEvents.sql, which is run under the ‘psql’ database utility called at the beginning of most volume matching scripts, analyzes the raw metadata and determines the number of points within 100 km of the radar that indicate ‘rain certain’ in the PR or DPR data, the percent of the 100-km range limit area covered by the satellite radar swath, and the percentages of this area indicating stratiform and convective rain type, and stores these values with identifying information in the database table ‘rainy100inside100’ for new rain events not yet tabulated.  The satellite/site specific rain events tallied in this table define and limit the data to be processed in most of the volume matching scripts, such that only data for rainy site overpasses are processed.  Completed matchup data files and their identifying metadata are cataloged in the database in the ‘geo_match_product’ table.  In general, the queries in the scripts will exclude those rainy site overpass events where the corresponding matchup output file is already tabulated in this table.

The volume matching scripts are the connection between the ‘gpmgv’ database, which contains information on site rain events and the satellite and ground radar data files associated with these events, and the IDL procedures that perform the volume matching of the satellite and ground radar data.  The scripts communicate the information from the database to the IDL matchup procedures in the form of a ‘control file’, a text file that lists the satellite and associated ground radar data files to be paired, along with metadata about the events (satellite ID, product type, product algorithm version, scan type, and orbit number; satellite overflight time; ground radar site ID, latitude, longitude, and elevation, etc.).  By convention, the control files are for rainy overpass events occurring on a single calendar date (UTC), and are specific to the type of satellite data product type to be processed.  The daily control files constitute the “what” part of the input to the IDL volume matching procedures.  The “how” and “where” parts are contained in a separate IDL ‘batch’ command file that configures the variable parameters that control the matchup algorithm and execute the IDL volume matching procedure specific to the satellite data type to be matched.

# Script Summaries – Baseline Matchups

The following tables describe the function and primary inputs and outputs of each volume matching script used in routine GPM Validation Network operations.

## do_DPR_GeoMatch.sh

In practice, do_DPR_GeoMatch.sh, which only processes one scan type for one DPR product type at a time, has been superseded by the more efficient matchup procedures provided by the scripts do_ GR_HS_MS_NS_GeoMatch.sh and do_DPR2GR_GeoMatch.sh.  Refer to Sections 2.4 and 2.5, below, for details.

| Main Script Name | do_DPR_GeoMatch.sh |
| --- | --- |
| Satellite Products Matched | 2ADPR, 2AKa, or 2AKu. Only one product type per run, as specified in input options to script. |
| Scan Type(s) Matched | HS,MS, or NS (2ADPR); HS or MS (2AKa); NS only (2AKu). Only one scan type per run, as specified in input option to script. |
| Control Files | DPR_files_sites4geoMatch.tag.txt, where ‘tag’ is a combination of product type, scan type, product version, and YYMMDD (the date of the matchup events), e.g., 2AKu.NS.V04A.161025 |
| Child Script Called | do_DPR_geo_matchup4date.sh, processes one date (one control file) at a time. Calls IDL to run the batch file commands, and tallies its success or failure in the database. |
| IDL Batch File Called | polar2dpr_V.bat, sets up matchup parameters and executes the polar2dpr IDL procedure, where “V” in the .bat file name is determined by the default or override value of the PARAMETER_SET variable. For instance, if PARAMETER_SET has the value 2, then the matchup scripts will locate and use the batch file named polar2dpr_2.bat in the invocation of IDL. |
| Output matchup files | GRtoDPR.rad.yymmdd.oo.vv.pp.ss.n_n.uu.nc.gz, where: GRtoDPR: Literal value, is the same regardless of whether the 2A product subtype is DPR, Ku, or Ka. rad: ID of ground radar, e.g. KAMX yymmdd: date of event oo: GPM orbit number vv: GPM product version, e.g., V04A pp: 2A product subtype, e.g., Ku ss: DPR scan type, e.g., NS n_n: Matchup file version, e.g. 1_21 for version 1.21 uu: Optional user-supplied addition to the filename nc: Literal value indicating a netCDF file type gz: Literal value, present if the file is gzip compressed |
| Log Files | do_DPR_GeoMatch.yymmdd.log (main script log, where yymmd here is the current calendar date when the script was started) do_DPR_geo_matchup4date.YYMMDD.log (child script log, where YYMMDD here is the date of the matchup events configured in the file) |

DESCRIPTION

-----------

Wrapper to do DPR-GR NetCDF geometric matchups for DPR 2A-[DPR|Ka|Ku] files

already received and cataloged, for cases meeting predefined criteria.  This

script drives volume matches between GPM DPR and ground radar (GR) data for

a single user-specified DPR scan type to produce the baseline "GRtoDPR"

matchup netCDF files.  This script queries the 'gpmgv' database to find rainy

site overpass events between a specified start and end date and assembles a

series of date-specific control files to run matchups for those dates.  For

each date, calls the child script do_DPR_geo_matchup4date.sh, which in turn

invokes IDL to generate the GRtoDPR volume match netCDF files for that day's

rainy overpass events as listed in the daily control file.  Ancillary output

from the script is a series of 'control files', one per day in the range of

dates to be processed, listing DPR and ground radar data files to be processed

for rainy site overpass events for that calendar date, as well as metadata

parameters related to the DPR and GR data and the site overpass events.

Volume matches are done for only one DPR 2A file type (2A-DPR, 2A-Ka, or

2A-Ku) at a time, and for only one scan type in that 2A data type.  The

2A-DPR contains 3 scan types (HS, MS, NS), the 2A-Ka contains 2 (HS, MS),

and the 2A-Ku only contains the NS scan type.

The script has logic to compute the start and end dates over which to attempt

volume match runs.  The end date is the current calendar day, and the start

date is 30 day prior to the current date.  These computed values exist to

support routine (cron-scheduled) runs of the script.  The computed values

are overridden in practice by specifying override values for the variables

'startDate' and 'endDate' in the main script itself, and these values must be

updated each time the script is to be (re)run manually.

Only those site overpasses within the user-specified date range which are

identified as 'rainy' will be configured in the daily control files to be run.

Event criteria are as defined in the table "rainy100inside100" in the "gpmgv"

database, whose contents are updated by an SQL query command file run in this

script as a default option.  Event definition includes cases where the DPR

indicates "rain certain" at 100 or more gridpoints within 100 km of the radar

within the 4km gridded 2A-DPR product.  See the SQL command file

${BIN_DIR}/'rainCases100kmAddNewEvents.sql'.

SYNOPSIS

--------

do_DPR_GeoMatch.sh [OPTION]...

OPTIONS/ARGUMENTS:

-----------------

-i INSTRUMENT_ID       Override default INSTRUMENT_ID (DPR) to the specified

INSTRUMENT_ID (Ka or Ku).  This determines which type of

2A product will be processed in the matchups.  STRING type.

-v PPS_VERSION         Override default PPS_VERSION to the specified PPS_VERSION.

This determines which version of the 2A product will be

processed in the matchups.  STRING type.

-p PARAMETER_SET       Override default PARAMETER_SET to the specified PARAMETER_SET.

This defines which version "V" of the IDL batch file

polar2dpr_V.bat is to be used in processing matchups.

This value also gets written to the geo_match_product table

in the gpmgv database as a descriptive attribute.  It's up

to the user to keep track of its use and meaning in relation

to the configured parameters in the polar2dpr_V.bat files.

See do_DPR_geo_matchup4date.sh for details. INTEGER type.

-w SWATH               Override the default DPR scan type (NS) to another

scan type (either MS or HS), as applicable to the current

INSTRUMENT_ID.  See the COMBO variable in the script.

-m GEO_MATCH_VERSION   Override default GEO_MATCH_VERSION to the specified

GEO_MATCH_VERSION.  This only changes if the IDL code that

produces the output netCDF file now produces a new or

different version of the matchup file.  If the value of

GEO_MATCH_VERSION is not the same as the version encoded in

the output netCDF filename an error is noted.  FLOAT type.

-k                     If specified, skip the step of updating the rain events in

the rainy100inside100 table.  Takes no argument value.

-f                     If specified, then instruct matchup programs to create

and overwrite any existing matchups for a date even if

the database says they have been run already (see NOTE).

Takes no argument value.

NOTE:  When running dates that might have already had DPR-GR matchup sets

run, the called script will skip these dates, as the 'appstatus' table

will say that the date has already been done.  Delete the entries

from this table where app_id='geo_match_IISS', where II is the value

of $INSTRUMENT and SS is the value of $SWATH, either for the date(s)

to be run, or for all dates.  EXCEPTION:  If script is called with the

-f option (e.g., "do_DPR_GeoMatch.sh -f"), then the status of prior

runs for the set of dates configured in the script will be ignored and

the matchups will be re-run, possibly overwriting the existing files.

EXAMPLES

--------

1) Run the script with all defaults.  Will run 2ADPR, version V04A, for NS scan type; will execute the SQL command to update the rainy100inside100 database table, and will skip any dates that have already been run. The range of dates that will be attempted is defined in the script by the startDate and endDate variables, which must be edited to reflect the desired data range.  The version of the batch file to be executed in IDL will be determined by the default value of PARAMETER_SET coded in the do_DPR_GeoMatch.sh script.  For instance, if PARAMETER_SET=2 in the script, then the batch file polar2dpr_2.bat will be run in IDL. From the command line, enter:

do_DPR_GeoMatch.sh

2) Run the script with all defaults as above, except use the matchup parameter set configured in the IDL batch file polar2dpr_1.bat. From the command line, enter:

do_DPR_GeoMatch.sh –p 1

3) Run the script to do matchups of the 2AKa HS scan type for the version V03D products, skip the SQL command to update the rainy100inside100 database table, and overwrite matchups for any dates that have already been run. The range of dates that will be attempted is defined in the script by the startDate and endDate variables, which must be edited to reflect the desired data range. From the command line, enter:

do_DPR_GeoMatch.sh –i Ka –w HS –v V03D –k -f

4) Clean out the entries in the appstatus table to allow a re-run of the 2A-DPR matchups for the NS scan type without having to do the –f option to force the run. From the command line, enter:

[morris@ds1-gpmgv ~]$ psql gpmgv

psql (8.4.20)

Type "help" for help.

to begin a psql session. At the psql prompt (gpmgv=>), enter and run the command:

gpmgv=> delete from appstatus where app_id = 'geo_match_DPRNS';

DELETE 593

to delete the entire desired set of entries (593 rows were deleted in this example, as shown by the status line output by psql).

To delete the appstatus entries to re-run a specific range of dates (for example, matching the range of dates defined by startDate 20160201 and endDate 20160228 in the script), enter and run the command:

gpmgv=> delete from appstatus where datestamp between ‘160201’ and ‘160228’ and app_id = 'geo_match_DPRNS';

Enter \q at the psql prompt to exit the ‘psql’ session and return to the shell prompt.

gpmgv=> \q

[morris@ds1-gpmgv ~]$

### The PARAMETER_SET variable and IDL Batch Files

Each matchup script type uses the IDL Batch file mechanism to configure and run the IDL matchup procedure that creates the output matchup files.  The IDL matchup procedures accept a number of mandatory and optional parameters that control various aspects of the matchup algorithm and affect the output data and file name.  By convention, the IDL batch file names used in the matchup scripts consist of a fixed part, and a variable part whose value is determined by the value of the PARAMETER_SET variable defined in, and exported from, the matchup script.  By convention, PARAMETER_SET is an integer value (0,1,2,3…).  The PARAMETER_SET value, whose default value can be overridden by the –p argument to the do_DPR_GeoMatch.sh script, determines which version of the IDL batch file will be run.  For instance, the name convention of the IDL batch files run by the do_DPR_GeoMatch.sh child script, do_DPR_geo_matchup4date.sh, is polar2dpr_V.bat, where the value of V in the batch file name takes the value of the PARAMETER_SET variable in the script.  If PARAMETER_SET has the value 2 in do_DPR_GeoMatch.sh, then do_DPR_geo_matchup4date.sh will format the batch file name polar2dpr_2.bat, will check for its existence, and if it exists will use it in the call to IDL.

It is expected that over time, it will be necessary to change the IDL configuration parameters in the batch file to meet the current matchup requirements or to re-run the matchups with a different configuration.  If the desired IDL configuration differs from those in any of the existing batch files such that the data content of the resulting matchup files is affected, then a new batch file with a new version number should be created.  To run matchups using these new configuration parameters either the default value of PARAMETER_SET in the do_DPR_GeoMatch.sh script should be modified to the new version value, or do_DPR_GeoMatch.sh should be passed the override value of the desired version by specifying the –p option as shown in the Example 2 in the previous section.   Old versions of batch files should be retained so that there is a record of how the matchup program was configured to create their matchup files, and because the PARAMETER_SET value is written to the ‘gpmgv’ database in the geo_match_product table that catalogs the matchup files.

Batch file parameters for the called IDL routine polar2dpr that would require a new version to be created if the parameter was changed include:

range_threshold_km

NC_NAME_ADD=ncnameadd

DPR_DBZ_MIN=dpr_dbz_min

DBZ_MIN=dBZ_min

DPR_RAIN_MIN=dpr_rain_min

DECLUTTER=declutter

DIR_BLOCK=dir_block

Typically, the value of NC_NAME_ADD is modified to reflect some reconfiguration of the other parameter values in the list so that it is easy to identify, by file name, the matchup files associated with that configuration.

## do_DPRGMI_GeoMatch.sh

By design, the IDL procedure called by this script performs volume matches of the ground radar to all scan types (MS and NS) and instruments (Ka and Ku) contained in the 2BDPRGMI product, and stores all the data subtypes in a single output netCDF file.  Thus, for a given 2BDPRGMI version and a given range of dates, it is only necessary to make one run of the do_DPRGMI_GeoMatch.sh script.

| Main Script Name | do_DPRGMI_GeoMatch.sh |
| --- | --- |
| Satellite Products Matched | 2BDPRGMI “Combined” product only. |
| Scan Type(s) Matched | MS and NS. Both scan types are matched up between the GR and the DPRGMI product, and both the Ka and Ku frequencies are matched up for the MS scan type, and all the scan types and frequencies are included in the output netCDF matchup file. |
| Control Files | COMB_files_sites4geoMatch.YYMMDD.txt, where YYMMDD is the date of the matchup events configured in the file, e.g., 161025. These control files are written to a product-version-specific subdirectory created in the scratch directory to support their identification and use by other scripts. |
| Child Script Called | do_DPRGMI_geo_matchup4date.sh, processes one date (one control file) at a time. Calls IDL to run the batch file commands, and tallies its success or failure in the database. |
| IDL Batch File Called | polar2dprgmi_V.bat, sets up matchup parameters and executes the polar2dprgmi procedure, where “V” in the .bat file name is determined by the default or override value of the PARAMETER_SET variable. For instance, if PARAMETER_SET has the value 2, then the matchup scripts will locate and use the batch file named polar2dprgmi_2.bat in the invocation of IDL. |
| Output matchup files | GRtoDPRGMI.rad.yymmdd.oo.vv.n_n.uu.nc.gz, where: GRtoDPRGMI: Literal value. rad: ID of ground radar, e.g. KAMX yymmdd: date of event oo: GPM orbit number vv: GPM product version, e.g., V04A n_n: Matchup file version, e.g. 1_21 for version 1.21 uu: Optional user-supplied addition to the filename nc: Literal value indicating a netCDF file type gz: Literal value, present if the file is gzip compressed |
| Log Files | doDPRGMIGeoMatch4NewRainCases.vv.yymmdd.log (main script log, where vv is as defined above, and yymmd here is the current calendar date when the script was started) do_DPRGMI_geo_matchup4date.YYMMDD.log (child script log, where YYMMDD here is the date of the matchup events configured in the file) |

DESCRIPTION

-----------

Wrapper to do DPRGMI-GR NetCDF geometric matchups for GPM 2B-DPRGMI files

already received and cataloged, for cases meeting predefined criteria.  This

script drives volume matches between DPRGMI and ground radar (GR) data for

both the MS and NS scan type and Ka and Ku instruments to produce the baseline

"GRtoDPRGMI" matchup netCDF files.  Queries the 'gpmgv' database to find rainy

site overpass events between a specified start and end date and assembles a

series of date-specific control files to run matchups for those dates.  For

each date, calls the child script do_DPRGMI_geo_matchup4date.sh, which in turn

invokes IDL to generate the GRtoDPRGMI volume match netCDF files for that day's

rainy overpass events as listed in the daily control file.  Ancillary output

from the script is a series of 'control files', one per day in the range of

dates to process, listing DPRGMI and ground radar data files to be processed

for rainy site overpass events for that calendar date, as well as metadata

parameters related to the DPRGMI and GR data and the site overpass events.

Volume matches are done for MS and NS scan types for Ku data, and MS only for

the Ka data.  Data for all scan types and frequencies are contained in the

GRtoDPRGMI netCDF matchup data files.  These files are not split out by scan

type and frequency as the 2A-DPR/Ka/Ku matchups are.

The script has logic to compute the start and end dates over which to attempt

volume match runs.  The end date is the current calendar day, and the start

date is 30 day prior to the current date.  These computed values exist to

support routine (cron-scheduled) runs of the script.  The computed values

are overridden in practice by specifying override values for the variables

'startDate' and 'endDate' in the main script itself, and these values must be

updated each time the script is to be (re)run manually.

Only those site overpasses within the user-specified date range which are

identified as 'rainy' will be configured in the daily control files to be run.

Event criteria are as defined in the table "rainy100inside100" in the "gpmgv"

database, whose contents are updated by an SQL query command file run in this

script as a default option.  Event definition includes cases where the DPR

indicates "rain certain" at 100 or more gridpoints within 100 km of the radar

within the 4km gridded 2A-DPR product.  See the SQL command file

${BIN_DIR}/'rainCases100kmAddNewEvents.sql'.

SYNOPSIS

--------

do_DPRGMI_GeoMatch.sh [OPTION]...

OPTIONS/ARGUMENTS:

-----------------

-v PPS_VERSION         Override default PPS_VERSION to the specified PPS_VERSION.

This determines which version of the 2B product will be

processed in the matchups.  STRING type.

-p PARAMETER_SET       Override default PARAMETER_SET to the specified PARAMETER_SET.

This defines which version "V" of the IDL batch file

polar2dprgmi_V.bat is to be used in processing matchups.

This value also gets written to the geo_match_product table

in the gpmgv database as a descriptive attribute.  It's up

to the user to keep track of its use and meaning in relation

to the configured parameters in the polar2dprgmi_V.bat files.

See do_DPRGMI_geo_matchup4date.sh for details. INTEGER type.

-m GEO_MATCH_VERSION   Override default GEO_MATCH_VERSION to the specified

GEO_MATCH_VERSION.  This only changes if the IDL code that

produces the output netCDF file now produces a new or

different version of the matchup file.  If the value of

GEO_MATCH_VERSION is not the same as the version encoded in

the output netCDF filename a fatal error occurs.  FLOAT type.

-k                     If specified, skip the step of updating the rain events in

the rainy100inside100 table.  Takes no argument value.

Option is disabled, but internally set to always skip the

updates under the assumption that updates were already

done by a prerequisite run of do_GR_HS_MS_NS_GeoMatch.sh.

-f                     If specified, then instruct matchup programs to create

and overwrite any existing matchups for a date even if

the database says they have been run already (see NOTE).

Takes no argument value.

-r                     If specified, then configure to run matchups to RHI

ground radar scans instead of the default PPI scans.

This capability is not yet supported, so this option

has no effect on the output.  Script will gracefully

exit if it is specified.

-c                     If specified, then configure to just create the control

files for the date range and defer running the volume

matches in IDL.  This helps support the mode where two

different machines might work simultaneously, running

volume matches for different dates.  See the related

script: do_DPRGMI_GeoMatch_from_ControlFiles.sh

NOTE:  When running dates that might have already had DPRGMI-GR matchup sets

run, the called script will skip these dates, as the 'appstatus' table

will say that the date has already been done.  Delete the entries

from this table where app_id='geo_match_COMB', either for the date(s)

to be run, or for all dates.  EXCEPTION:  If script is called with the

-f option (e.g., "do_DPRGMI_GeoMatch.sh -f"), then the status of prior

runs for the set of dates configured in the script will be ignored and

the matchups will be re-run, possibly overwriting the existing files.

EXAMPLES

--------

1) Run the script with all defaults.  Will run 2BDPRGMI, version V04A.  Will execute the SQL command to update the rainy100inside100 database table, and will skip any dates that have already been run. The range of dates that will be attempted is defined in the script by the startDate and endDate variables, which must be edited to reflect the desired data range.  The version of the batch file to be executed in IDL will be determined by the default value of PARAMETER_SET coded in the do_DPRGMI_GeoMatch.sh script.  For instance, if PARAMETER_SET=2 in the script, then the batch file polar2dprgmi_2.bat will be run in IDL. From the command line, enter:

do_DPRGMI_GeoMatch.sh

2) Run the script with all defaults as above, except use the matchup parameter set configured in the IDL batch file polar2dprgmi_1.bat. From the command line, enter:

do_DPRGMI_GeoMatch.sh –p 1

3) Run the script to do matchups of the version V03D products, skip the SQL command to update the rainy100inside100 database table, and overwrite matchups for any dates that have already been run. The range of dates that will be attempted is defined in the script by the startDate and endDate variables, which must be edited to reflect the desired data range.

do_DPRGMI_GeoMatch.sh -v V03D -k -f

4) Run the script with all defaults, but including the –c option, which will instruct the script to produce the daily control files for the range of dates with rain events, but will skip the call to the child script do_DPRGMI_geo_matchup4date.sh, which actually runs the matchups.  The range of dates that will be attempted is defined in the script by the startDate and endDate variables, which must be edited to reflect the desired data range.

do_DPRGMI_GeoMatch.sh -c

### do_DPRGMI_GeoMatch_from_ControlFiles.sh

There is a variation on the use of the do_DPRGMI_GeoMatch.sh script that divides its functions up into two scripts:  the first produces a series of daily control files but does not call the child script to do matchups, and a second script that identifies a series of DPRGMI matchup files produced by the first and calls the child script to run the matchups defined in the daily control files.  The first function is accomplished by calling do_DPRGMI_GeoMatch.sh (Section 2.2) with the ‘-c’ option specified on the command line.  The second function is accomplished by running this script, do_DPRGMI_GeoMatch_from_ControlFiles.sh, configured to look in the product-version-specific subdirectory to locate the version and dates of control files previously created by the do_DPRGMI_GeoMatch.sh script, and then calls the child script do_DPRGMI_geo_matchup4date.sh to run the GR-DPRGMI matchups in the same manner as do_DPRGMI_GeoMatch.sh.

The main difference is that, unlike do_DPRGMI_GeoMatch.sh, the internal function catalog_to_db in this script does not access the PostGRESQL database ‘gpmgv’, it only prepares the output netCDF file cataloging metadata intended for the database and writes it to a text file for later, manual loading to the database.  In that way, this script can successfully run and produce matchups on a separate machine that does not have access to the database.  For this mode of operation to succeed without errors will require the addition of an option to the shared child script do_DPRGMI_geo_matchup4date.sh to instruct it to also not interact with the database.

| Main Script Name | do_DPRGMI_GeoMatch_from_ControlFiles.sh |
| --- | --- |
| Satellite Products Matched | 2BDPRGMI “Combined” product only. |
| Scan Type(s) Matched | MS and NS. Both scan types are matched up between the GR and the DPRGMI product, and both the Ka and Ku frequencies are matched up for the MS scan type, and all the scan types and frequencies are included in the output netCDF matchup file. |
| Control Files | COMB_files_sites4geoMatch.YYMMDD.txt, where YYMMDD is the date of the matchup events configured in the file, e.g., 161025. These control files are located in a product-version-specific subdirectory in the scratch directory to support their identification and use by this script. |
| Child Script Called | do_DPRGMI_geo_matchup4date.sh, processes one date (one control file) at a time. Calls IDL to run the batch file commands, and tallies its success or failure in the database. |
| IDL Batch File Called | polar2dprgmi_V.bat, sets up matchup parameters and executes the polar2dprgmi procedure, where “V” in the .bat file name is determined by the default or override value of the PARAMETER_SET variable. For instance, if PARAMETER_SET has the value 2, then the matchup scripts will locate and use the batch file named polar2dprgmi_2.bat in the invocation of IDL. |
| Output matchup files | GRtoDPRGMI.rad.yymmdd.oo.vv.n_n.uu.nc.gz, where: GRtoDPRGMI: Literal value. rad: ID of ground radar, e.g. KAMX yymmdd: date of event oo: GPM orbit number vv: GPM product version, e.g., V04A n_n: Matchup file version, e.g. 1_21 for version 1.21 uu: Optional user-supplied addition to the filename nc: Literal value indicating a netCDF file type gz: Literal value, present if the file is gzip compressed |
| Log Files | doDPRGMIGeoMatch4NewRainCases.vv.yymmdd.log (main script log, where vv is as defined above, and yymmd here is the current calendar date when the script was started). do_DPRGMI_geo_matchup4date.YYMMDD.log (child script log, where YYMMDD here is the date of the matchup events configured in the file) |

DESCRIPTION

-----------

When the script do_DPRGMI_GeoMatch.sh is run with the '-c' option, it will

query the gpmgv database for dates/times of rainy DPR or GR events and write

daily control files used as input to do the DPRGMI-GR geometry matching for

events with data, but will stop short of doing the actual matchups.  This

script will look at a series of these control files for a range of dates

internally defined by 'dateStart' and 'dateEnd', locate the matching control

files for the $SAT_ID/$ALGORITHM/$PPS_VERSION and date combination, and

for each such control file, will call do_DPRGMI_geo_matchup4date.sh which

then runs IDL to do the GR/DPRGMI matchups.  This mode allows more than one

IDL matchup process to run at a time, on the same host or a different host,

against a set of non-overlapping dates. This is useful to speed up

reprocessing a complete set of matchups for a new version of GPM or ground

radar data.

Completed geometry match files are cataloged in a delimited text file that

can be loaded into the 'gpmgv' database table 'geo_match_product', present

on ds1-gpmgv.  This file is written to in append mode in the function

catalog_to_db() contained in this script file, so it always grows until it is

manually deleted, hopefully following a successful manual loading to the

database.  Unlike in other scripts, catalog_to_db() only prepares the data to

load to the database, and does not do the actual loading step.  This allows

this script to be run on a 2nd machine that does not have database access.

See SQL_BIN2 in the catalog_to_db() documentation.

SYNOPSIS

--------

do_DPRGMI_GeoMatch_from_ControlFiles.sh [OPTION]...

OPTIONS/ARGUMENTS:

-----------------

-v PPS_VERSION         Override default PPS_VERSION to the specified PPS_VERSION.

This determines which version of the 2B product will be

processed in the matchups.  STRING type.

-p PARAMETER_SET       Override default PARAMETER_SET to the specified PARAMETER_SET.

This defines which version "V" of the IDL batch file

polar2dprgmi_V.bat is to be used in processing matchups.

This value also gets written to the geo_match_product table

in the gpmgv database as a descriptive attribute.  It's up

to the user to keep track of its use and meaning in relation

to the configured parameters in the polar2dprgmi_V.bat files.

See do_DPRGMI_geo_matchup4date.sh for details. INTEGER type.

-m GEO_MATCH_VERSION   Override default GEO_MATCH_VERSION to the specified

GEO_MATCH_VERSION.  This only changes if the IDL code that

produces the output netCDF file now produces a new or

different version of the matchup file.  If the value of

GEO_MATCH_VERSION is not the same as the version encoded in

the output netCDF filename a fatal error occurs.  FLOAT type.

-f                     If specified, then instruct matchup programs to create

and overwrite any existing matchups for a date even if

the database says they have been run already (see NOTE).

Takes no argument value.

-r                     If specified, then configure to run matchups to RHI

ground radar scans instead of the default PPI scans.

This capability is not yet supported, so this option

has no effect on the output.  Script will gracefully

exit if it is specified.

## do_GMI_GeoMatch.sh

| Main Script Name | do_GMI_GeoMatch.sh |
| --- | --- |
| Satellite Products Matched | 2AGPROF, with 1CRXCAL where available. Data from GPM, TRMM, or any constellation satellite may be used, despite the fact that the scripts, control files, and IDL programs are named according to the GPM GMI instrument. In practice, only GPM GMI and TRMM TMI have matching ground radar data. Satellite and instrument IDs are specified as options to script; defaults are GPM and GMI. |
| Scan Type(s) Matched | Not applicable, only the conical scan is made, and footprints are spatially matched for all variables. |
| Control Files | GMI_files_sites4geoMatch.YYMMDD.txt, where YYMMDD is the date of the matchup events configured in the file, e.g., 161025 |
| Child Script Called | do_GMI_geo_matchup4date.sh, processes one date (one control file) at a time. Calls IDL to run the batch file commands, and tallies its success or failure in the database. |
| IDL Batch File Called | polar2gmi.bat, sets up matchup parameters and executes the polar2gmi procedure |
| Output matchup files | GRtoGPROF.sat.mi.rad.yymmdd.oo.vv.n_n.uu.nc.gz, where: GRtoGPROF: Literal value. sat: ID of satellite, e.g., GPM, METOPA mi: ID of microwave instrument, e.g., GMI, MHS, SSMIS rad: ID of ground radar, e.g. KAMX yymmdd: date of event oo: GPM orbit number vv: GPM product version, e.g., V04A n_n: Matchup file version, e.g. 1_21 for version 1.21 uu: Optional user-supplied addition to the filename nc: Literal value indicating a netCDF file type gz: Literal value, present if the file is gzip compressed |
| Log Files | do_satmi_GeoMatch.vv.yymmdd.log (main script log, where sat, mi, and vv are as defined above, and yymmd here is the current calendar date when the script was started). sat and mi are merged in the file name, e.g.: do_GPMGMI_GeoMatch.V03B.160414.log do_GMI_geo_matchup4date.YYMMDD.log (child script log, where YYMMDD here is the date of the matchup events configured in the file). Currently always labeled as GMI, regardless of the satellite whose data are being processed. |

DESCRIPTION

-----------

Wrapper to do GPROF-GR NetCDF geometric matchups for 2A-GPROF and 1C-R-XCAL

files in the GV data system, for cases meeting predefined criteria.  This

script drives volume matches between GPROF and ground radar (GR) data to

produce the baseline "GRtoGPROF" matchup netCDF files.  Queries the 'gpmgv'

database to find rainy site overpass events between a specified start and end

date and assembles a series of date-specific control files to run matchups for

those dates.  By default, the GPM GMI 2A-GPROF and 1C-R-XCAL data are used in

the volume matching, but this script and its called procedures have the

capability to do matchups against these product types for any constellation

satellite, since the file formats are the same.  In practice this is not

feasible, since the matching ground radar data typically are not available

except for the GPM and TRMM satellites.

For each date, calls the child script do_GMI_geo_matchup4date.sh, which

invokes IDL to generate the GRtoGPROF volume match netCDF files for that day's

rainy overpass events as listed in the daily control file.  Ancillary output

from the script is a series of 'control files', one per day in the range of

dates to be processed, listing 2A-GPROF, 1C-R-XCAL (if available), and ground

radar data files to be processed for rainy site overpass events for that

calendar date, as well as metadata parameters related to the GPROF and GR

data and the site overpass events.

Volume matches are done using the 2A-GPROF data products, and will also

include equivalent blackbody temperature data from the matching (i.e., same

satellite, orbit, orbit subset, and version) 1C-R-XCAL product, if available.

Completed geometry match files are cataloged in the 'gpmgv' database table

'geo_match_product'.

The script has logic to compute the start and end dates over which to attempt

volume match runs.  The end date is the current calendar day, and the start

date is 30 day prior to the current date.  These computed values exist to

support routine (cron-scheduled) runs of the script.  The computed values

are overridden in practice by specifying override values for the variables

'startDate' and 'endDate' in the main script itself, and these values must be

updated each time the script is to be (re)run manually.

Only those site overpasses within the user-specified date range which are

identified as 'rainy' will be configured in the daily control files to be run.

Event criteria are as defined in the table "rainy100inside100" in the "gpmgv"

database, whose contents are updated by an SQL query command file run in this

script as a default option.  Event definition includes cases where the DPR

indicates "rain certain" at 100 or more gridpoints within 100 km of the radar

within the 4km gridded 2A-DPR product.  See the SQL command file

${BIN_DIR}/'rainCases100kmAddNewEvents.sql'.

SYNOPSIS

--------

do_GMI_GeoMatch.sh [OPTION]...

OPTIONS/ARGUMENTS:

-----------------

-s SAT_ID              Override default SAT_ID (GPM) to the specified SAT_ID.

This, along with INSTRUMENT_ID, determines which type

of 2AGPROF product will be processed in the matchups.

STRING type.

-i INSTRUMENT_ID       Override default INSTRUMENT_ID (GMI) to the specified

INSTRUMENT_ID that pertains to the specified SAT_ID.

This determines which type of 2AGPROF product will be

processed in the matchups.  STRING type.

-v PPS_VERSION         Override default PPS_VERSION to the specified PPS_VERSION.

This determines which version of the 2A/1C products will be

processed in the matchups.  STRING type.

-p PARAMETER_SET       Override default PARAMETER_SET to the specified PARAMETER_SET

This tracks which version of IDL batch file was used in

processing matchups, when changes are made to the batch file.

This value does not actually control anything, but it gets

written to the geo_match_product table in the gpmgv database

as a descriptive attribute.  It's up to the user to keep track

of its use and meaning.  INTEGER type.

-m GEO_MATCH_VERSION   Override default GEO_MATCH_VERSION to the specified

GEO_MATCH_VERSION.  This only changes if the IDL code that

produces the output netCDF file now produces a new or

different version of the matchup file.  If the value of

GEO_MATCH_VERSION is not the same as the version encoded in

the output netCDF filename a fatal error occurs.  FLOAT type.

-k                     If specified, skip the step of updating the rain events in

the rainy100inside100 table.  Takes no argument value.

-f                     If specified, then instruct matchup programs to create

and overwrite any existing matchups for a date even if

the database says they have been run already (see NOTE).

Takes no argument value.

NOTE:  When running dates that might have already had GPROF-GR matchup sets

run, the called script will skip these dates, as the 'appstatus' table

will say that the date has already been done.  Delete the entries

from this table where app_id='geo_match_gmi', either for the date(s)

to be run, or for all dates.  EXCEPTION:  If script is called with the

-f option (e.g., "do_GMI_GeoMatch.sh -f"), then the status of prior

runs for the set of dates configured in the script will be ignored and

the matchups will be re-run, possibly overwriting the existing files.

## do_GR_HS_MS_NS_GeoMatch.sh

To run matchups between DPR and GR for all available DPR 2A products (2ADPR, 2AKa, 2AKu) and scan types (HS, MS, NS) would require 6 separate runs of the do_DPR_GeoMatch.sh script, since only one product and scan type is handled at a time by this legacy script and IDL procedure.  The GR data files would need to be copied, uncompressed, open, and read each time the script is run, and this is a slow process now that the GR data files are so large.  To alleviate some of this processing time, do_ GR_HS_MS_NS_GeoMatch.sh and its called IDL procedure reads the 2ADPR product which contains all 3 DPR scan types, reads the GR data once and volume matches the GR fields to each of the 3 DPR scan types, and saves the volume-matched GR data only, for all 3 scan types, in an intermediate site/version/orbit-specific netCDF file.  The volume-matched GR data in these intermediate files can then be merged with the product- and scan-specific DPR data, volume-matched to the GR data in the follow-on script do_DPR2GR_GeoMatch.sh, run separately at a later time to produce the baseline GRtoDPR matchup netCDF files.  All the spatial information needed to volume-match the DPR data for each scan type to the GR data is contained in the intermediate netCDF files, alleviating the need to re-read the GR data files in the merging process.  The assumption here is that the HS and MS (NS) scan locations are the same between the 2ADPR and 2AKa (2AKu) products for a given orbit and version.

| Main Script Name | do_ GR_HS_MS_NS_GeoMatch.sh |
| --- | --- |
| Satellite Products Matched | 2ADPR only. Performs GR-only volume matching to DPR footprint locations for all of the 3 DPR scan types. A run of this script should be followed up with a sequence of runs of do_DPR2GR_GeoMatch.sh for the same range of dates. Product- and scan-type-specific 2ADPR, 2AKa, or 2AKu volume matching to the GR data is performed by do_DPR2GR_GeoMatch.sh and merged with previously volume-matched GR data output by this script to produce the baseline GRtoDPR netCDF matchup files containing both volume-matched DPR and GR data. |
| Scan Type(s) Matched | HS,MS, and NS. GR data are matched to all three scan types per run. |
| Control Files | DPR_files_sites4geoMatch.tag.txt, where ‘tag’ is a combination of product type, scan type (hard-coded as ‘All3’), product version, and YYMMDD (the date of the matchup events), e.g., 2AKu.All3.V04A.161025 |
| Child Script Called | do_GR_HS_MS_NS_geo_matchup4date.sh, processes one date (one control file) at a time. Calls IDL to run the batch file commands, and tallies its success or failure in the database. |
| IDL Batch File Called | polar2dpr_hs_ms_ns_V.bat, sets up matchup parameters and executes the polar2dpr_hs_ms_ns procedure, where “V” in the .bat file name is determined by the default or override value of the PARAMETER_SET variable. For instance, if PARAMETER_SET has the value 2, then the matchup scripts will locate and use the batch file named polar2dpr_hs_ms_ns_2.bat in the invocation of IDL. See note in section 2.5 regarding coordination of polar2dpr_hs_ms_ns and dpr2gr_prematch batch files. |
| Output matchup files | GRtoDPR_HS_MS_NS.rad.yymmdd.oo.vv.n_n.uu.nc.gz, where: GRtoDPR_HS_MS_NS: Literal value. rad: ID of ground radar, e.g. KAMX yymmdd: date of event oo: GPM orbit number vv: GPM product version, e.g., V04A n_n: Matchup file version, e.g. 1_0 for version 1.0 uu: Optional user-supplied addition to the filename nc: Literal value indicating a netCDF file type gz: Literal value, present if the file is gzip compressed |
| Log Files | do_GR_HS_MS_NS_GeoMatch.yymmdd.log (main script log, where yymmd here is the current calendar date when the script was started) do_GR_HS_MS_NS_geo_matchup4date.YYMMDD.log (child script log, where YYMMDD here is the date of the matchup events configured in the file) |

DESCRIPTION

-----------

Wrapper to do GR geometric matchups to HS, MS, and NS scans in 2A-DPR files

already received and cataloged, for cases meeting predefined criteria.  This

script drives volume matches of GR data to DPR footprint locations for all

of the DPR scan types.  The DPR-matched GR data are written to event-specific

"GRtoDPR_HS_MS_NS" netCDF files that hold the GR matchups for all three types

of DPR scans in a single file.  These matchup files are specific to GR site,

GPM orbit, PPS processing version of the input 2A-DPR data, and matchup file

definition version (currently 1.0).  Output of this script and its called

processes consists of a set of intermediate netCDF files containing ground

radar data only, volume-matched to all three (HS, MS, NS) DPR scan types.

Ancillary output is a series of 'control files', one per day in the range of

dates to be processed, listing DPR and ground radar data files to be processed

for rainy site overpass events for that calendar date, as well as metadata

parameters related to the DPR and GR data and the site overpass events.

A run of this script should be followed up with one or more runs of

do_DPR2GR_GeoMatch.sh for the same range of dates to complete the creation of

baseline GRtoDPR netCDF files containing both volume-matched DPR and GR data.

The script has logic to compute the start and end dates over which to attempt

volume match runs.  The end date is the current calendar day, and the start

date is 30 day prior to the current date.  These computed values exist to

support routine (cron-scheduled) runs of the script.  The computed values

are overridden in practice by specifying override values for the variables

'startDate' and 'endDate' in the main script itself, and these values must be

updated each time the script is to be (re)run manually.

Only those site overpasses within the user-specified date range which are

identified as 'rainy' will be configured in the daily control files to be run.

Event criteria are as defined in the table "rainy100inside100" in the "gpmgv"

database, whose contents are updated by an SQL query command file run in this

script as a default option.  Event definition includes cases where the DPR

indicates "rain certain" at 100 or more gridpoints within 100 km of the radar

within the 4km gridded 2A-DPR product.  See the SQL command file

${BIN_DIR}/'rainCases100kmAddNewEvents.sql'.

SYNOPSIS

--------

do_GR_HS_MS_NS_GeoMatch.sh [OPTION]...

OPTIONS/ARGUMENTS:

-----------------

-v PPS_VERSION         Override default PPS_VERSION to the specified PPS_VERSION.

This determines which version of the 2ADPR product will be

processed in the matchups.  STRING type.

-p PARAMETER_SET       Override default PARAMETER_SET to the specified PARAMETER_SET.

This defines which version "V" of the IDL batch file

polar2dpr_hs_ms_ns_V.bat is to be used in processing matchups.

This value also gets written to the geo_match_product table

in the gpmgv database as a descriptive attribute.  It's up

to the user to keep track of its use and meaning in relation

to the configured parameters in the polar2dpr_hs_ms_ns_V.bat

files. See do_GR_HS_MS_NS_geo_matchup4date.sh for details.

INTEGER type.

-m GEO_MATCH_VERSION   Override default GEO_MATCH_VERSION to the specified

GEO_MATCH_VERSION.  This only changes if the IDL code that

produces the output netCDF file now produces a new or

different version of the matchup file.  If the value of

GEO_MATCH_VERSION is not the same as the version encoded in

the output netCDF filename a fatal error occurs.  FLOAT type.

-k                     If specified, skip the step of updating the rain events in

the rainy100inside100 table.  Takes no argument value.

-f                     If specified, then instruct matchup programs to create

and overwrite any existing matchups for a date even if

the database says they have been run already (see NOTE).

Takes no argument value.

NOTE:  When running dates that have already had GR->DPR matchup sets run,

the child script do_GR_HS_MS_NS_geo_matchup4date.sh will skip

processing for these dates, as its query of the 'appstatus' table

will say that the date has already been done.  Delete the entries

from this table where app_id='geo_match_GRx3', either for the date(s)

to be run, or for all dates.  EXCEPTION:  If script is called with the

-f option (e.g., "do_GR_HS_MS_NS_GeoMatch.sh -f"), then the status of

prior runs for the set of dates configured in the script is ignored and

the matchups will be re-run, possibly overwriting the existing files.

## do_DPR2GR_GeoMatch.sh

This script is a required follow-on to the do_ GR_HS_MS_NS_GeoMatch.sh script described in the preceding section.  Once do_ GR_HS_MS_NS_GeoMatch.sh has completed for a given range of dates, do_DPR2GR_GeoMatch.sh is run for each desired 2A product subtype (DPR, Ka, or Ku) to compete the final-output GRtoDPR matchup files for every scan type present within that product type.  This requires editing the startDate and endDate values in the do_DPR2GR_GeoMatch.sh script to match the date range used in the run of do_ GR_HS_MS_NS_GeoMatch.sh.

| Main Script Name | do_DPR2GR_GeoMatch.sh |
| --- | --- |
| Satellite Products Matched | 2ADPR (default), 2AKa, or 2AKu. Only one 2A product subtype is processed per run, either for the default ‘DPR’ subtype or as specified in ‘-i’ input option to script, e.g.: do_DPR2GR_GeoMatch.sh –i Ku |
| Scan Type(s) Matched | HS,MS, and NS (2ADPR); HS and MS (2AKa); NS (2AKu). Called IDL procedure automatically performs a separate matchup for each scan type that applies to the selected 2A satellite product subtype, resulting in separate output matchup files with differing ‘ss’ filename components. |
| Control Files | DPR_files_sites4geoMatch.tag.txt, where ‘tag’ is a combination of product type, scan type (hard-coded as ‘All’), product version, and YYMMDD (the date of the matchup events), e.g., 2AKu.All.V04A.161025 |
| Child Script Called | do_DPR2GR_geo_matchup4date.sh, processes one date (one control file) at a time. Calls IDL to run the batch file commands, and tallies its success or failure in the database. |
| IDL Batch File Called | dpr2gr_prematch_V.bat, sets up matchup parameters and executes the dpr2gr_prematch procedure, where “V” in the .bat file name is determined by the default or override value of the PARAMETER_SET variable. For instance, if PARAMETER_SET has the value 2, then the matchup scripts will locate and use the batch file named dpr2gr_prematch_2.bat in the invocation of IDL. |
| Output matchup files | GRtoDPR.rad.yymmdd.oo.vv.pp.ss.n_n.uu.nc.gz, where filename components are the same as defined in Section 2.1, for do_DPR_GeoMatch.sh. |
| Log Files | do_DPR2GR_GeoMatch.yymmdd.log (main script log, where yymmd here is the current calendar date when the script was started) do_DPR2GR_geo_matchup4date.YYMMDD.log (child script log, where YYMMDD here is the date of the matchup events configured in the file) |

Note:

As a follow-on script to the do_ GR_HS_MS_NS_GeoMatch.sh script described in the preceding section, it is important that the IDL parameters configured in the batch files that drive the matchups are configured identically for parameters in common between the two IDL matchup procedures.  In particular, if a different NC_NAME_ADD value is specified in the two batch files, then the IDL dpr2gr_prematch procedure run by do_DPR2GR_GeoMatch.sh will fail to find the corresponding GR-only intermediate netCDF data file output by the polar2dpr_hs_ms_ns procedure run by do_ GR_HS_MS_NS_GeoMatch.sh, and the final GRtoDPR netCDF matchup files will not be able to be produced.  In general, this should be accomplished by defining matching parameters for the two batch files of the same version (see IDL Batch File Called in the tables above), and specifying the same PARAMETER_SET option value when running the two scripts.

It is also required that the same PPS_VERSION option is used in paired runs of these two scripts.

DESCRIPTION

-----------

Wrapper to do DPR-GV NetCDF geometric matchups for DPR 2A-[DPR|Ka|Ku] files

already received and cataloged, for cases meeting predefined criteria.  This

script drives volume matches of DPR data to GR data that has already been

volume matched to DPR for all of the DPR scan types.  The previously matched

GR data are contained in "GRtoDPR_HS_MS_NS" netCDF files, which are read and

merged with the volume matched DPR data to produce the baseline "GRtoDPR"

matchup netCDF files.  This script queries the 'gpmgv' database to find rainy

site overpass events between a specified start and end date and assembles a

series of date-specific control files to run matchups for those dates.  For

each date, calls the child script do_DPR2GR_geo_matchup4date.sh, which in turn

invokes IDL to generate the DPRtoGR volume match netCDF files for that day's

rainy overpass events as listed in the daily control file.  Ancillary output

from the script is a series of 'control files', one per day in the range of

dates to be processed, listing DPR and ground radar data files to be processed

for rainy site overpass events for that calendar date, as well as metadata

parameters related to the DPR and GR data and the site overpass events.

The script has logic to compute the start and end dates over which to attempt

volume match runs.  The end date is the current calendar day, and the start

date is 30 day prior to the current date.  These computed values exist to

support routine (cron-scheduled) runs of the script.  The computed values

are overridden in practice by specifying override values for the variables

'startDate' and 'endDate' in the main script itself, and these values must be

updated each time the script is to be (re)run manually.

Only those site overpasses within the user-specified date range which are

identified as 'rainy' will be configured in the daily control files to be run.

Event criteria are as defined in the table "rainy100inside100" in the "gpmgv"

database, whose contents are updated by an SQL query command file run in this

script as a default option.  Event definition includes cases where the DPR

indicates "rain certain" at 100 or more gridpoints within 100 km of the radar

within the 4km gridded 2A-DPR product.  See the SQL command file

${BIN_DIR}/'rainCases100kmAddNewEvents.sql'.

SYNOPSIS

--------

do_DPR2GR_GeoMatch.sh [OPTION]...

OPTIONS/ARGUMENTS:

-----------------

-i INSTRUMENT_ID       Override default INSTRUMENT_ID (DPR) to the specified

INSTRUMENT_ID (Ka or Ku).  This determines which type of

2A product will be processed in the matchups.  STRING type.

-v PPS_VERSION         Override default PPS_VERSION to the specified PPS_VERSION.

This determines which version of the 2A product will be

processed in the matchups.  STRING type.

-p PARAMETER_SET       Override default PARAMETER_SET to the specified PARAMETER_SET.

This defines which version "V" of the IDL batch file

dpr2gr_prematch_V.bat is to be used in processing matchups.

This value also gets written to the geo_match_product table

in the gpmgv database as a descriptive attribute.  It's up

to the user to keep track of its use and meaning in relation

to the configured parameters in the dpr2gr_prematch_V.bat

files. See do_DPR2GR_geo_matchup4date.sh for details.

INTEGER type.

-m GEO_MATCH_VERSION   Override default GEO_MATCH_VERSION to the specified

GEO_MATCH_VERSION.  This only changes if the IDL code that

produces the output netCDF file now produces a new or

different version of the matchup file.  If the value of

GEO_MATCH_VERSION is not the same as the version encoded in

the output netCDF filename a fatal error occurs.  FLOAT type.

-f                     If specified, then instruct matchup programs to create

and overwrite any existing matchups for a date even if

the database says they have been run already (see NOTE).

Takes no argument value.

NOTE:  When running dates that might have already had DPR-GV matchup sets

run, the child script do_DPR2GR_geo_matchup4date.sh will skip

processing for these dates, as a check of the 'appstatus' table

will say that the date has already been done.  Delete the entries

from this table where app_id='geo_matchIIAll', where II is the value

of $INSTRUMENT and the rest of the characters (geo_match and All) are

fixed and literal values, for the date(s) to be run, or for all dates.

EXAMPLE SQL:  DELETE FROM appstatus WHERE app_id='geo_matchKuAll';

EXCEPTION:  If script is called with the -f option

(e.g., "do_DPR2GR_GeoMatch.sh -f"), then the status of prior runs

for the set of dates configured in the script will be ignored and

the matchups will be re-run, possibly overwriting the existing files.

The SWATH variable has the fixed value "All" in this script, as the

IDL procedure dpr2gr_prematch.pro automatically runs matchup for all

the scan types present in the 2A product type being processed.

PRECONDITIONS

-------------

Since this script relies on the existence of previously-volume-matched ground

radar data in "GRtoDPR_HS_MS_NS" netCDF files, do_GR_HS_MS_NS_GeoMatch.sh

must already have been configured and successfully run over the start and

end date range configured in this script.  Neither this script nor its child

script do_DPR2GR_geo_matchup4date.sh check for the pre-existence of these

GRtoDPR_HS_MS_NS netCDF files.

# Script Summaries - Non-routine and legacy matchup scripts

## do_DPR_GeoMatch_NPOL_MD_rules.sh

This script is a variation of the script do_DPR_GeoMatch.sh described in Section 2.1, but processes GPM DPR matchups only for the NPOL ground radar.  It contains additional features to deal with the mobility and RHI scan capabilities of the NPOL radar.  Ground radar data files for the NPOL radar are all named, stored in the file system, and cataloged in the database under the site ID "NPOL", regardless of where the radar is installed and operating. Satellite overpass and rain event criteria for the NPOL radar are provided in the PPS CT (coincidence Table) files and cataloged in the ‘gpmgv’ database under location-specific site IDs depending on where the NPOL radar has been installed and operated; for example, "NPOL_MD" for the Wallops installation or “NPOL_WA” for the OLYMPEX field campaign.  This script specifically deals with overpass events where the NPOL was installed at Wallops (NPOL_MD).  A separate but related script, do_DPR_GeoMatch_NPOL_OLY_rules.sh, drives the DPR matchups to NPOL for the period where NPOL was installed at the OLYMPEX field site (NPOL_WA).

The database VIEW (a predefined set of queries that acts like a virtual database table) named ‘collate_npol_md_1cuf’ (‘collate_npol_wa_1cuf’ in the case of NPOL_WA) attempts to automatically reconcile the naming difference between the variable, satellite-related site ID and the fixed ID ‘NPOL’ used for the ground radar files/directories and their database entries.  This view can fail to associate the correct NPOL ground radar file to an NPOL rain event listed in the database for one of two reasons:  either the NPOL radar scan time of the desired GR scan type (PPI or RHI) is outside the time range considered to be “coincident” between the satellite overpass and the GR volume, or some other disconnect between the database entries exists that causes the VIEW to not return a valid row of data or to identify an NPOL volume different from the best one to be matched to the satellite data.  In normal NPOL operations there may be multiple volume scans taken in a mix of PPI and RHI mode for a given GPM overflight.

To allow the user to manually reconcile the NPOL ground radar file to be used for the event, the main script pauses as each date’s control file is prepared and brings the control file up in an editor to allow the user to make changes or corrections to the NPOL radar file partial pathname (or the missing-file placeholder value “no_1CUF_file”) listed in the control file before proceeding with the volume matching procedures.

A simple argument (-r) to the script configures it to attempt DPR-GR matchups using NPOL volume scans taken in RHI scan mode.  NPOL data files taken in RHI mode have a “.rhi” component to the file name to distinguish them from files for volume scans taken in PPI scan mode.  At the IDL level a different ‘.bat’ command file and different IDL program are called when processing matchups for RHI scans.

| Main Script Name | do_DPR_GeoMatch_NPOL_MD_rules.sh (do_DPR_GeoMatch_NPOL_OLY_rules.sh) |
| --- | --- |
| Satellite Products Matched | 2ADPR, 2AKa, or 2AKu. Only one product type per run, as specified in input options to script. |
| Scan Type(s) Matched | HS,MS, or NS (2ADPR); HS or MS (2AKa); NS only (2AKu). Only one scan type per run, as specified in input option to script. |
| Control Files | DPR_files_sites4geoMatch.tag.txt, where ‘tag’ is a combination of product type, scan type, product version, and YYMMDD (the date of the matchup events), e.g., 2AKu.NS.V04A.161025 |
| Child Script Called | do_DPR_geo_matchup4date.sh, processes one date (one control file) at a time. Calls IDL to run the batch file commands, and tallies its success or failure in the database. |
| IDL Batch File Called | In default (PPI) matchup mode: polar2dpr.bat, sets up matchup parameters and executes the polar2dpr procedure. In RHI matchup mode: : rhi2dpr.bat, sets up matchup parameters and executes the rhi2dpr procedure. |
| Output matchup files | GRtoDPR.rad.yymmdd.oo.vv.pp.ss.n_n.uu.nc.gz, where: GRtoDPR: Literal value, is the same regardless of whether the 2A product subtype is DPR, Ku, or Ka. rad: ID of ground radar, e.g. NPOL_MD yymmdd: date of event oo: GPM orbit number vv: GPM product version, e.g., V04A pp: 2A product subtype, e.g., Ku ss: DPR scan type, e.g., NS n_n: Matchup file version, e.g. 1_21 for version 1.21 uu: Optional user-supplied addition to the filename nc: Literal value indicating a netCDF file type gz: Literal value, present if the file is gzip compressed |
| Log Files | do_DPR_GeoMatch.yymmdd.log (main script log, where yymmd here is the current calendar date when the script was started). Same log file name convention as for do_DPR_GeoMatch.sh do_DPR_geo_matchup4date.YYMMDD.log (child script log, where YYMMDD here is the date of the matchup events configured in the file) |

DESCRIPTION

-----------

Wrapper to do DPR-GR geometric matchups for 2ADPR/2AKa/2AKu and NPOL 1CUF

files already received and cataloged, for cases meeting predefined criteria.

This version of the script uses the 'gpmgv' database VIEW collate_npol_md_1cuf

to find the matching 1CUF ground radar data file for the event.  These files

are all cataloged under the site ID "NPOL", while the overpass and rain event

criteria are cataloged under the site ID "NPOL_MD".  The view attempts to

account for the site ID disconnect between the different database tables

involoved in generating the control files, but does not always succeed in

finding the correct matching NPOL 1CUF file for the event.  To handle this,

this "special" script brings up each new control file in the editor 'gedit'

and waits while the user makes any required manual changes to the control file

to replace missing or incorrect NPOL 1CUF file placeholders with the correct

matching 1CUF filenames.  Once editing is complete and the editor is dismissed

the script will allow the user to choose either to proceed with the matchups

for this control file or skip the matchup step and go to the next date.

Criteria are as defined in the query which is run to update the table

"rainy100inside100" in the "gpmgv" database.  Includes cases where the DPR

indicates "rain certain" at 100 or more gridpoints within 100 km of the NPOL

radar within a 4km gridded 2A-Ku product.  See the SQL command file

rainCases100kmAddNewEvents.sql'.

SYNOPSIS

--------

do_DPR_GeoMatch_NPOL_MD_rules.sh [OPTION]...

OPTIONS/ARGUMENTS:

-----------------

-i INSTRUMENT_ID       Override default INSTRUMENT_ID (DPR) to the specified

INSTRUMENT_ID (Ka or Ku).  This determines which type of

2A product will be processed in the matchups.  STRING type.

-v PPS_VERSION         Override default PPS_VERSION to the specified PPS_VERSION.

This determines which version of the 2A product will be

processed in the matchups.  STRING type.

-p PARAMETER_SET       Override default PARAMETER_SET to the specified PARAMETER_SET

This tracks which version of IDL batch file was used in

processing matchups, when changes are made to the batch file.

This value does not actually control anything, but it gets

written to the geo_match_product table in the gpmgv database

as a descriptive attribute.  It's up to the user to keep track

of its use and meaning.  INTEGER type.

-w SWATH               Override the default DPR scan type (NS) to another

scan type (either MS or HS), as applicable to the current

INSTRUMENT_ID.  See the COMBO variable in the script.

-m GEO_MATCH_VERSION   Override default GEO_MATCH_VERSION to the specified

GEO_MATCH_VERSION.  This only changes if the IDL code that

produces the output netCDF file now produces a new or

different version of the matchup file.  If the value of

GEO_MATCH_VERSION is not the same as the version encoded in

the output netCDF filename an error is noted.  FLOAT type.

-k                     If specified, skip the step of updating the rain events in

the rainy100inside100 table.  Takes no argument value.

-f                     If specified, then instruct matchup programs to create

uncataloged matchups for rain events on a date, even if

the 'appstatus' table in the database indicates the date's

matchups for the type have been run already (see NOTE).

Takes no argument value.

-r                     If specified, then query the database for GR files with

RHI scans and instruct child script to call a version of

matchup programs to generate RHI volume matches to the

DPR data, rather than the default PPI matchups.

Takes no argument value.

-n                     Like -f option, but instructs the queries that build the

control file to include all rain events even if the

database table 'geo_match_product' indicates that a

matching output netCDF file for the event already exists

(event-by-event override, versus date-by-date override).

Takes no argument value.

NOTE:  When running dates that might have already had DPR-GR matchup sets

run, the called script will skip these dates, as the 'appstatus' table

will say that the date has already been done.  Delete the entries

from this table where app_id='geo_match_IISS', where II is the value

of $INSTRUMENT and SS is the value of $SWATH, either for the date(s)

to be run, or for all dates.  EXCEPTION:  If script is called with the

-f option (e.g., "do_DPR_GeoMatch_NPOL_MD_rules.sh -f"), then the status

of prior runs for the dates configured in the script will be ignored and

the matchups will be re-run, possibly overwriting the existing files.

- By default, processes matchups for the site ID 'NPOL_MD' only.

This cannot be changed unless a different VIEW than

collate_npol_md_1cuf is coded in SQL and defined in the ‘gpmgv’

database.

## do_DPRtoGMI_OrbitMatch.sh

This is an incomplete script, still in development.  It is intended to produce a listing of 2A-GPROF GMI files for orbit subsets that have matching rain events at overflown ground radar sites as input to the IDL dpr2gmi_driver procedure, which in turn calls the IDL function dpr2gmi to do the individual matchups for each orbit subset product (file).  The IDL dpr2gmi function performs spatial matching between surface rain rate estimates in the GMI GPROF product and the near-surface rain rate estimates in the matching 2A-[DPR/Ka/Ku] and (optionally) the 2B-DPRGMI products along the GPM orbit track, and (optionally) stores all the matched rainrate data in an output netCDF file.

While the ground radar product itself is not involved in the spatial matching process, this script still queries the database for rainy overpass events at the GR sites to assure that there is a minimum coverage of observed precipitation in the DPR swath for a given GPM orbit.  Only one of the DPR products (2A-DPR, 2A-Ku, or 2A-Ka) and one of the scan types (HS, MS, or NS) is matched to the GMI rain rate data in a run of dpr2gmi_driver.

For now, dpr2gmi_driver is run directly in IDL, and the necessary configuration parameters are manually input to dpr2gmi_driver.  Its output is considered to be an experimental product and is not routinely produced for the Validation Network data set.  However, this script can be used to produce a control file

| Main Script Name | do_DPRtoGMI_OrbitMatch.sh |
| --- | --- |
| Satellite Products Matched | 2ADPR, 2AKa, or 2AKu. Only one product type per run, as specified in input options to script. Optionally includes the matching 2BDPRGMI “Combined” product in the matchups (identified and handled internally by the IDL code). |
| Scan Type(s) Matched | HS,MS, or NS (2ADPR); HS or MS (2AKa); NS only (2AKu). Only one scan type per run, as specified in input option to script. If HS, then the 2BDPRGMI product is excluded from the matchup by the IDL matchup procedure. |
| Control Files | GMI_orbit_files_sites4geoMatch.YYMMDD.txt, where YYMMDD is the date of the matchup events configured in the file, e.g., 161025 |
| Child Script Called | TBD. |
| IDL Batch File Called | TBD. The IDL procedure to be called within the future batch file is dpr2gmi_driver. |
| Output matchup files | DPRtoGMI_NN.yyyymmdd.oo.vv.UU_pp_ss_VV_DD.dkm.nc.gz, where: DPRtoGMI_NN: Literal value. yyyymmdd: date of event oo: GPM orbit number vv: GMI-specific product version, e.g., V03B UU: Orbit subset label, e.g., CONUS pp: 2A product subtype, e.g., Ku ss: DPR scan type, e.g., NS VV: 2A-[DPR/Ka/Ku]-specific product version, e.g., V04A DD.dkm: Averaging radius of DPR samples around the GMI footprint center point, where DD.d is the radial distance in km in the floating point format nn.n, and km is a literal value. nc: Literal value indicating a netCDF file type gz: Literal value, present if the file is gzip compressed |
| Log Files | doDPRtoGMI_OrbitMatch.vv.yymmdd.log (main script log, where vv is as defined above, and yymmd here is the current calendar date when the script was started) TBD.log (child script log, TBD) |

DESCRIPTION

-----------

TBD.

The following sections describe the function and primary inputs and outputs of the legacy (TRMM-specific) volume matching scripts used in the prototype and GPM-era Validation Network operations.

## do_PR_GeoMatch.sh

The legacy script do_PR_GeoMatch.sh processes matchups between the TRMM PR and ground radars using multiple PR products in TRMM HDF4 files.  Unlike the all-in-one GPM DPR products, TRMM PR variables of interest are distributed among multiple product types, e.g.: 1C21 (measured Z, land/ocean flag, surface clutter blanking), 2A23 (bright band/QC fields), and 2A25 (corrected Z, rain type, 3-D and surface rain rate, BB height).  If the matching 2B31 file is available, it is processed to include the combined PR/TMI surface rain rate in the matchups.  The 1C21 and 2A25 products are mandatory, while matchups are allowed to proceed if the matching 2A23 or 2B31 files are not available.

There is no new file format in the GPM era for the PR data files, although the file naming convention was changed to meet the GPM filename convention beginning on 3/1/2014.  There is about a 2-week overlap between files in the old naming convention (located under /data/gpmgv/prsubsets) in ‘flat’ product-specific directories, and those with the new naming convention and PPS-like directory structures (located under /data/gpmgv/orbit_subset/TRMM).

| Main Script Name | do_PR_GeoMatch.sh |
| --- | --- |
| Satellite Products Matched | 1C21, 2A23, 2A25, and 2B31. Only the 1C21 and 2A25 are mandatory. |
| Scan Type(s) Matched | Not applicable, PR does not have multiple scans. |
| Control Files | PR_files_sites4geoMatch.yymmdd.txt, where yymmdd is the date of the matchup events. |
| Child Script Called | do_geo_matchup4date.sh, processes one date (one control file) at a time. Calls IDL to run the batch file commands, and tallies its success or failure in the database. |
| IDL Batch File Called | polar2pr.bat, sets up matchup parameters and executes the polar2pr procedure |
| Output matchup files | GRtoPR.rad.yymmdd.oo.vv.n_n.uu.nc.gz, where: GRtoPR: Literal value. rad: ID of ground radar, e.g. KAMX yymmdd: date of event oo: TRMM orbit number vv: TRMM product version (6 or 7 for the original TRMM product naming convention; V06 or V07 for TRMM products generated in the GPM era.) n_n: Matchup file version, e.g. 1_21 for version 1.21 uu: Optional user-supplied addition to the filename nc: Literal value indicating a netCDF file type gz: Literal value, present if the file is gzip compressed |
| Log Files | doGeoMatch4NewRainCases.SUBSET.yymmdd.log (main script log, where yymmd here is the current calendar date when the script was started, and SUBSET is the orbit subset ID of the PR data files used in the matchup.) do_geo_matchup4date.YYMMDD.log (child script log, where YYMMDD here is the date of the matchup events configured in the file) |

DESCRIPTION

-----------

Wrapper to do PR-GR NetCDF geometric matchups for 1C21/2A25/2B31/1CUF files

already received and cataloged, for cases meeting predefined criteria.

Criteria are as defined in the query that is run to update the table

"rainy100inside100" in the "gpmgv" database.  Includes cases where the PR

indicates "rain certain" at 100 or more gridpoints within 100 km of the radar

within the 4km gridded 2A-25 product.  See the SQL command file

'rainCases100kmAddNewEvents.sql'.

SYNOPSIS

--------

do_PR_GeoMatch.sh [OPTION]...

OPTIONS/ARGUMENTS:

-----------------

-v PPS_VERSION         Override default PPS_VERSION to the specified PPS_VERSION.

This determines which version of the TRMM products will be

processed in the matchups.  STRING type.

-p PARAMETER_SET       Override default PARAMETER_SET to the specified PARAMETER_SET

This tracks which version of IDL batch file was used in

processing matchups, when changes are made to the batch file.

This value does not actually control anything, but it gets

written to the geo_match_product table in the gpmgv database

as a descriptive attribute.  It's up to the user to keep track

of its use and meaning.  INTEGER type.

-u SUBSET              Override default (CONUS) orbit subset of data to process to the

specified subset.  This also controls which ground radar sites

will be included in the processing.

NOTE:  When running dates that might have already had PR-GR matchup sets

run, the called script will skip these dates, as the 'appstatus' table

will say that the date has already been done.  Delete the entries

from this table where app_id='geo_match_PR', either for the date(s) to

be run, or for all dates.

## doTMIGeoMatch4NewRainCases.sh

The legacy script doTMIGeoMatch4NewRainCases.sh processes matchups between the TRMM TMI and ground radars using pre-GPM-era TRMM 2A-12 HDF4 files.  Starting with the GPM era, TRMM TMI products were produced and/or reprocessed in the 2A-GPROF HDF5 format, whose matchups are handled by the do_GMI_GeoMatch.sh script (Section 2.3) with options specified to instruct TRMM TMI products to be processed. There is about a 2-week overlap between files in the old naming convention (located under /data/gpmgv/prsubsets/2A12) in a ‘flat’ directory, and those with the new naming convention and PPS-like directory structures (located under /data/gpmgv/orbit_subset/TRMM).

| Main Script Name | doTMIGeoMatch4NewRainCases.sh |
| --- | --- |
| Satellite Products Matched | 2A12 |
| Scan Type(s) Matched | Not applicable, TMI does not have multiple scans. |
| Control Files | TMI_files_sites4geoMatch.yymmdd.txt, where yymmdd is the date of the matchup events. |
| Child Script Called | do_TMI_geo_matchup4date.sh, processes one date (one control file) at a time. Calls IDL to run the batch file commands, and tallies its success or failure in the database. |
| IDL Batch File Called | polar2tmi.bat, sets up matchup parameters and executes the polar2tmi procedure |
| Output matchup files | GRtoTMI.rad.yymmdd.oo.vv.n_n.uu.nc.gz, where: GRtoPR: Literal value. rad: ID of ground radar, e.g. KAMX yymmdd: date of event oo: TRMM orbit number vv: TRMM product version (6 or 7 for the original TRMM product naming convention) n_n: Matchup file version, e.g. 1_21 for version 1.21 uu: Optional user-supplied addition to the filename nc: Literal value indicating a netCDF file type gz: Literal value, present if the file is gzip compressed |
| Log Files | doTMIGeoMatch4NewRainCases.Vvv.yymmdd.log (main script log, where yymmd here is the current calendar date when the script was started, vv is the version of TMI data files used in the matchup (6 or 7), and all other characters are fixed, literal values. do_TMI_geo_matchup4date.YYMMDD.log (child script log, where YYMMDD here is the date of the matchup events configured in the file) |

DESCRIPTION

-----------

Query gpmgv database for dates/times of rainy PR or GR events and assemble

command string to do the TMI-GR geometry matching for events with data.

Completed geometry match files are cataloged in the 'gpmgv' database table

'geo_match_products'.

SYNOPSIS

--------

doTMIGeoMatch4NewRainCases.sh

OPTIONS/ARGUMENTS:

-----------------

None. All reconfiguration is done by editing values of variables internal to the script.

NOTE:  When running dates that might have already had TMI-GR matchup sets

run, the called script will skip these dates, as the 'appstatus' table

will say that the date has already been done.  Delete the entries

from this table where app_id='geo_match_tmi', either for the date(s) to

be run, or for all dates.